Marine Panel PC Series

Intel® Core™ i5 Dual-core Processor

Intel® Core™ i5FDeuaatl-ucroeresProcessor

(Optional Stand Kit)

- 19” and 24” Fanless Panel PC with 4th Generation Intel® Core™ i5 Dual-core Processor

- -15°C ~ +55°C Wide Temperature and IP66 Protection with Flat-bezel Projected Capacitive Touchscreen

- Excellent Visual Performance

  - Three independent displays

  - Full OSD function configuration

  - 0%~100% full range dimming

  - 178°/178° wide viewing angles

  - Optical bonding improves visibility under bright light (optional)

- Isolation Protection

  - RS-232/422/485 isolated serial ports

  - Isolated 18V ~ 36V DC input

  - 2 x Isolated CAN-bus 2.0B

- iRIS Remote Management Module

Specifications

S24A-QM87

S19A-QM87

| Model |  | S19A-QM87 | S24A-QM87 |
| --- | --- | --- | --- |
| LCD | LCD Size | 19” | 24” |
|  | Panel Type | PMVA | AMVA |
|  | Max. Resolution | 1280 x 1024 (5:4) | 1920 x 1080 (16:9) |
|  | Contrast Ratio | 2000 : 1 | 5000 : 1 |
|  | Brightness (cd/m²) | 300 | 300 |
|  | LCD Color | 16.7M | 16.7M |
|  | Pixel Pitch (um) | 294 x 294 | 276.75 x 276.75 |
|  | Viewing Angles (H-V) | 178° / 178° | 178° / 178° |
|  | Backlight MTBF (HRs) | 50000 | 50000 |
| Touchscreen & Controller |  | EEX3188 (10 point) , 6H | EEX3188 (10 point) , 6H |
| System | SBC Model | SMB-QM87-R10 |  |
|  | Chipset | Intel® QM87 |  |
|  | CPU | Intel® 22nm 4th generation Mobile Core™ i5-4400E 2.7GHz processor |  |
|  | RAM | 4GB 1600MHz DDR3 SDRAM (2GB*2) Two 204-pin 1600/1333 MHz dual-channel DDR3 SDRAM support up to 16GB |  |
| Input Interfaces | I/O Ports & Switch | 2 x USB 2.0 1 x Isolated 18V ~ 36V DC 3-pin terminal block 2 x USB 3.1 Gen 1 (5Gb/s) 1 x Power button 1 x VGA 1 x PS2 (through Y-type cable supporting KB/MS) 1 x DVI-D 2 x CAN-bus 2.0B, 3-pin terminal block (2.5 kv isolation protection) 1 x HDMI 1 x DB-9 RS-232 (non-isolated) , 1 x Mic-in 4 x DB-9 RS-232/422/485 (2.5 kv isolation protection) 1 x Line-out 2 x RJ-45 GbE LAN with teaming support - x Line-in (2KV isolation protection) - x Antenna SMA hole (reserved) |  |
|  | Expansion | 2 x Full-size PCIe Mini slot, 1 x iRIS-2400 slot (iRIS remote management module) |  |
| Ethernet Controller |  | GbE1: Intel® I217LM with Intel® AMT 9.0 support GbE2: Intel® I210-AT PCIe controller |  |
| Sensor |  | Ambient light sensor (0% ~ 100%) |  |
| OSD |  | Menu, brightness down, brightness up, LCD on/off by cap-sensor (graphics engine: Genesis STDP6038) |  |
| Storage |  | 2 x 2.5” SSD bay with RAID 0/1 function 1 x Accessible CFast socket |  |
| LED Indicator |  | Power (power on: green; power off: orange)/storage (red, blinking)/IPMI (blue) |  |
| Power Requirement |  | Isolated 24V DC (18~31.2 V DC), screw-type 3-pin terminal block |  |
| Operating Temperature |  | -15°C ~ 55°C (5°F ~ 131°F) |  |
| Storage Temperature |  | -20°C ~ 60°C (-4°F ~ 140°F) |  |
| Humidity |  | 5% to 95% RH |  |
| Thermal Design |  | Fanless |  |
| Watchdog Timer |  | Supports 1~255 sec. system reset |  |
| Certifications |  | EMC: CE, FCC Safety: DNV, IEC 60945 4th, IACS-E10, IEC 61174 IP rating: IP66 compliant front panel and IP22 compliant rear cover |  |
| Housing |  | Front aluminum sheet metal (Black C) |  |
| Cut-out Dimensions (L x W) |  | 442 mm x 373 mm | 576 mm x 358 mm |
| Dimensions (L x W x H) |  | 463 mm x 394 mm x 113 mm | 592 mm x 374 mm x 113 mm |
| Net Weight (kg) |  | 9.91 | 13.03 |
| Gross Weight (kg) |  | 13.38 | 17.19 |
| Vibration and Shock |  | IEC 60945 / DNV 2.4 compliant / IACS-E10 |  |
| Power Supply (DC Input) |  | DC Input ATX Power Supply - P/N: 041D710-00-101-RS - 150W power supply - Input: 24V DC, 8-4A (max.) - Output (max.): 12V @13A |  |
| Power Consumption |  | 112W | 117W |

# Ordering Information

## w w w . i ei w or l d .c om

| P/N | Description |
| --- | --- |
| S19A-QM87i-i5/PC/4G-R11 | 19” 300cd/m² SXGA marine panel PC with Intel® Mobile Core™ i5-4400E 2.7GHz CPU, 2*2GB DDR3 RAM, projected capacitive touch screen, isolated 18-36V DC, iRIS-2400 supported, R11 |
| S24A-QM87i-i5/PC/4G-R11 | 24” 300cd/m² FHD marine panel PC with Intel® Mobile Core™ i5-4400E 2.7GHz CPU, 2*2GB DDR3 RAM, projected capacitive touch screen, isolated 18-36V DC, iRIS-2400 supported, R11 |

Options

| Item | Part No. | Description |
| --- | --- | --- |
| Panel Mount Kit | PK-S19A-R10 | For S19A |
|  | PK-S24A-R10 | For S24A |
| Desktop Stand | STAND-S24A-R10 | For S19A & S24A |
| Ceiling Mount Kit | CEILMT-S24-R10 | For S19A/S24A |
| HDMI Lockable Kit | HDMI-LK-R10 | Universal HDMI Locking Adapter |
| iRIS Module | iRIS-2400-R10 | IPMI 2.0 adapter card with AST2400 BMC chip for DDR3 SO-DIMM socket interface |

Packing List

| Item | Q’ty | Remark |
| --- | --- | --- |
| S19A-QM87 or S24A-QM87 | 1 |  |
| Screw Pack | 1 | Including necessary screws |
| PS/2 Cable | 1 | Round cable; PS/2 cable |
| Wire Strain Band | 3 | Wire strain band |

STAND-S24A-R10	CEILMT-S24-R10

CFast Slot HDMI

GbE LAN 1

Power Switch

24V DC

2 x USB 2.0

1 x PS/2

DVI-D

VGA

2 x CAN-bus 2.0B

Line-in

Line-out Mic-in

Combo RJ-45: GbE LAN 2 /IPMI

Display System

COM 1: RS-232 2 x USB 3.1 Gen 1 (5Gb/s)

Four OSD Keys by Cap-sensor

(Menu, Brightness down, Brightness up, LCD on/off)

Ambient Light Sensor

LED Indicators: Power

HDD IPMI

2 x Antenna SMA hole

COM 2

COM 3

COM 4

COM 5

COM 2~COM 5: RS-232/422/485

S19A-QM87 Dimensions

(Unit: mm)

S24A-QM87 Dimensions

(Unit: mm)